Coolmay 3G

PLC/PLC All-in-one

USB programming port drive installation steps in WIN7&WIN10

Shenzhen Coolmay Technology Co.,Ltd

V12.15

Note: Please colse the digital signature first when you using os WIN10, then manually install Mini type B USB programming port drive.

The steps of closing digital signature in WIN10 are as below:

---After setting third step, you will enter the restart setting interface. It is recommended to take photos from step from 4-8 or remember the setting method before you operation next.

- Click the Start menu button --- click on Settings

- 2、Enter the settings window，click update and security（windows update、recovery、backup）；

3、Click recovery，then click "Start Now" in the advanced startup, after rebooting, you will enter the following window.

4、After rebooting, pop up the first window as below and click "Troubleshoot"

5、Then enter the second window, click advanced options

- 6、Then click ‘Startup Settings’

- After entering the startup settings window, you need to click "Restart" to activate the software;

- After restarting the startup settings interface, press the number key 7 or F7 on the keyboard to disable the driver enforcement signature.

- After drive enforcement signature prohibited and computer restarts, then manually boot the Mini B type USB programming port driver

The Mini type B USB programming port WIN7 system and WIN10 system (execute after digital signature have been closed) drive need to be manually installed, the steps are as follows: unzip《3G series PLC&PLC all-in-one USB programming port drive_WIN7 os》， cover file of ECUsbd.inf as below to C:\MELSEC\Easysocket\USBDrivers（The path can be found according to the installation path of the PLC programming software).

- After the operation is completed, connect the USB programming cable to the computer and open the device manager. Unknown devices will appear in other devices. Right-click to update the driver software. The operation steps are as follows:

In the above figure, the path of “browsing the driver files on the computer” should be corresponding to the USBDrivers in the PLC programming software installation path (the same as the coverage path of the first step ECUsbd.inf file), as shown below, click

When the USB driver is successfully installed, one of the device manager's Universal Serial Bus controllers will be added, as shown below.

After the installation finish, you can use the usb cable to download the PLC

program.